Brian, 

Sorry I no longer have the instructions. The instructions used to be located on a website that is no longer active. I still have my 99 with the VFR bars installed. Installing the VFR bars was a real simple unbolting and removing the original bars and putting the VFR bars on. The only thing that required modification was the master cylinders. The hydraulic lines that go to the master cylinder are long enough to reach the new higher location when the VFR bars are installed but the lines need to be rotated downward a bit and into a more natural drop position. There are tabs on the master cylinders that hold the hydraulic lines in a fixed locked position that prevent you from rotating the lines. In order to rotate the lines into a new more natural drop position with the VFR bars installed you need to cut the tab off to allow the hydraulic lines to rotate. The master cylinder and the tabs are a soft cast material and easy to cut. I used a dremel tool to cut the tab but you could use a hack saw or other cutting tools. I found the dremel easy since it was a somewhat tight space to get at. Once you have the tabs cut off you will want to have some rags to catch any of the hydraulic fluid that might spill out when you loosen the banjo joint at the master cylinder to rotate the hydraulic lines. When loosening the banjo joint you just need to get it loose enough to rotate them. You want them snug so you don't introduce air or have fluid leak out. When I did it no fluid leaked out but I still had rags around just in case. The hydraulic fluid can damage painted surfaces. Don't squeeze the brake or clutch levers while loosening and adjusting the hydraulic lines our you could force fluid out and introduce air into the lines. So you want the banjo joint to be fairly snug and just want it loose enough to rotate the hydraulic line into a more natural drop position while not introducing air. Then tighten the lines back up and you should be good to go. I remember that it took me about a half an hour for the entire process and that included gathering tools pulling the bike out into my driveway for better lighting and space and doing all of the removal of the old and installation of the VFR bars. I probably spent another half an hour admiring my handiwork. Ha Ha Hope this helps. Do you already have a set of the VFR bars? 


[image: image1.png]


 Originally Posted by Brira73 
 

Hi
I wouldt like to do this mod. Couldt you please send me the install instructions?
Thank you

Kind regards
Brian Rasmussen, Denmark

